PHASE 1:

Sun-Synchronous Orbit (SSO) at 10:30 LTAN

Need consistent time -> consistent lighting -> Easier for image comparison

Use SSO:

- Force orbit to turn at rate earth orbits sun

- Around 1 degree a day

- Intentionally fly at specific inclination (around 97-98 degrees)

- Takes advantage of J2 perturbation

Consistent 10:30 LTAN means passing over at the same time. 10:30am also provides enough shadow for depth perception of satellites whilst maintaining good sunlight.

Allows meeting SNR greater than 100.

J2 Perturbation: Gravitational effects caused by Earths oblateness

Select inclination and altitude to counteract J2 effect

5-day Revisit

Revisit under 5 days for target latitudes between 40 and 80 degrees.

Satellite can roll up to 45 degrees off-nadir -> don’t need single line on ground.

Width of swath dependent on altitude. Lower altitudes = Narrower strip.

To ensure any target can be seen between 40 and 8- degrees every 5 days, orbital ground tracks must be laid down tightly together.

Summary: At lower altitudes, geometry restricts physical width of access swath. To guarantee a 5 day revisit at the 40-80 degree latitude band, specific altitude must be selected to prevent gaps in off-nadir coverage.

Slant Range vs Altitude

Must guarantee a 1m GSD at 45-degree off-nadir angle

Altitude is straight line to ground, but worst-case imaging occurs at 45 degrees to the side.

As Earth is a sphere, ground curves away at the edges of the view.

Angled distance to the target is the Slant Range

I.e. if you fly at an altitude of 300km for example, you cant size for a 300km distance, you need to correct for the curvature.

Sizing the Aperture (D)

Calculate for optical aperture for a 1m GSD at 550nm Wavelength.

Physical limit of how small an object a telescope can resolve governed by

θ=1.22 λD

Wavelength is set at 550nm. Therefore If you increase the altitude, then the Diameter of the aperture has to increase.

GSD=θ*S

∴GSD=1.22 λD*S

Where S is the Slant Range.

Therefore rearranging for Aperture Diameter:

D=1.22*λ*SGSD

Mass Scaling

Governed by:  OTA Mass=146 D1.27

If you double the diameter you quadruple the area.

Needs an exponentially stiffer backing structure

Higher altitudes require larger aperture to maintain 1m resolution.

SNR

Focal number between 3-15 times.

Focal Ratio (F-Number): fD

Low F-number means highly concentrated light -> bright image -> high SNR (Fast Telescope)

High F-Number means less concentrated, light is spread out before hitting sensor -> darker image -> lowers SNR (Slow Telescope)

In the CW:

- Focal length has no effect on SNR as GSD is set to 1.0

- Focal length has no effect on Mass and volume

- Must be in range of 6D to 30D

- GSD=r*pf

- Therefore pixel size and focal length simply scale relative to eachother

Coverage between 40 and 80 degree latitude area.

Enforced by calculating the field of regard at each altitude of the system based on the slant range. Compare this to the gap between tracks on the orbit. If within the 5 day period the field of regard covers the tracks then it is valid.

Note: single day repeated orbits cover the same tracks, and thus need enormous fields of regard.

Pixel Size

The lecture on optical systems states that:

θpixel≤θ2

i.e. the pixel size should be half of the GSD minimum to prevent aliasing

Sensor trade space created, input the valid altitude and aperture chosen and pick any of the valid number of pixels vs pitch width.