PHASE 3

Presentation

Industry Trends: Air-Breathing Electric Propulsion – Currently at a low Technology Readiness Level.

Model Presentation: Simulink Screenshot + Flowchart to represent MATLAB Code

Physical Configuration:

- Payload – pointing Nadir

- Solar Panels – At poles/Wrapped

- ADCS – Near CoM

- Thrusters – Mounted on Wake face

- Antennas – As close to Nadir as possible

Critical Design Dependencies:

- Aref dictates Drag

- Drag dictates Thrust Power needed

- Thrust power dictates battery size and Solar Panel Size

  - Battery size affects overall mass

  - Solar Panel Size affects overall mass

- Solar Panel size affects Aref

Safety Features:

- During Extreme Storm (ap = 300), sat can enter safe mode, feathering its solar arrays to reduce drag and cutting off thrust, leaving only basic electrical systems active (40W draw). F10.7 = 250 and F10.7A =200

- During Extreme conditions nominal requirements are suspended

- After 72 hours (average storm length) a climb and re-orbit is initiated

  - Historical Space Weather Benchmarks

    - Carrington Event

    - Modern Extreme Storms

    - Main CME density spikes settle after 3-4 days (mybib sources)

- Climb back to altitude -> Payload off (60W saved) apply this power to ion thruster.

- Use a sunlit-only climb strategy

Nominal Values:

- F10.7 = 150

  - Mission takes place in 2026 – Maximum Solar Cycle

  - Peak Sunspot activity of 115

  - F10.7=68.84+0.67*SSN≈146

- Ap = 15

  - Between 8-15 is “Unsettled”, maximum unsettled characteristic

SNR Justification

- QSo = 500

  - Total is around 1360Wm-2 so take 40%

- Albedo = 0.2

  - Sized to allow imaging of relatively dark, low contrast ground targets that are prevalent in Europe and North America especially cities and forests.

- Topt=0.85

  - Small percentage of light absorption or scattering

  - Between 80%-90% is standard industry practice

- Qeff=0.9

  - Measures how good the sensor is at converting incoming light into digital signal

  - Industry trends improved efficiency to around 90% from the standard of 50% in the past. Using modern Backside-Illuminated CMOS Sensors.

Configuration:

Patch Antenna on Nadir Face – Need Cuboid Bus for free corner space.

Thrusters on the Wake face – Counteracts Aerodynamic Drag

Nadir: Payload, Patch Antenna, Sun Sensor

Ram: ATOX Shielding, Sun Sensor

Wake: Ion Thrusters, Sun Sensor, Launch Vehicle Adapter

Port and Starboard: Solar Arrays, Sun Sensor, Star Trackers?

Zenith: Thermal Radiators, Sun Sensor, Star Tracker, Magnetometer

Further Drag Reduction

Curved Dome on Ram:

Time Delay Integration

By utilising multiple rows of pixels, artificially stacks exposure time without blurring.

SNRnew=SNRbase*√NTDI

Considerations

- Crab Angle – Diagonal Blur, constant yaw steering from ADCS

- Jitter – Vibrations amplified as shutter kept open

  - Would need ultra-smooth ADCS

  - Would need heavy vibration isolation

- Dynamic Velocity Mismatch

  - Ground Speed not constant

  - Rolling changes geometric distance

  - Must constantly calculate relative ground velocity and adjust clock rate

- In Reality VERY Expensive

Beta Angle

Angle between Suns radiation vector and satellites plane, drive by LTAN and Earth seasonal tilt.

Base offset:

- Earth rotates 15 degrees per hour

- 10:30am is 1.5 hours away from solar noon

- 1.5*15=22.5∘

Earth is tilted on its axis by 23.5 degrees.

SSO at 96.8

βmax≈LTAN Offset+Solar Declination*sinInclination

Roughly equal to 46 degrees maximum.

Workflow

Phase 1:

Step 1: Find Valid Altitudes

- Define Altitude Range

- Within Range, find SSO Repeating Ground Tracks

- Filter for less than or equal to 5 day revisit time

- Based on Off-Nadir Roll angle, find if Field of Regard > Gap between Ground Tracks

  - Need Slant Range

- This Gives valid altitudes!

Step 2: Size Aperture

For Each Altitude

- Use Rayleigh Criterion to find initial Aperture Diameter

- Check if SNR > 100

- If not, increase diameter slightly and repeat above step

- If SNR > 100, get length ( 2 * D )

- Calculate OTR Mass (146 * D^1.27)

Step 3: Orbit Characteristics

For Each Altitude

- Calculate Orbital Period

- Calculate Orbital Velocity

- Calculate Inclination (Proof of SSO)

- Calculate Eclipse Fraction based on β=0 (Worst Case)

- Calculate Eclipse Time

PHASE 2

Step 1:

- Base on Aperture, construct models at average altitudes (I used 400km) in SOLIDWORKS for ADBSat to get CD Values (Assumed 1m^2 solar array each side)

  - Assuming Model is equal to size of telescope + clearance

- Find ISP, Power and Efficiency values for different Thruster Types

- Define Margins (Dry Mass, Propellant, Power, Battery Capacity)

Step 2: Main Loop – Consists of a for loop with a nested for loop with another  nested for loop with a final While loop (4 layers)

- Outer Loop

  - Set to run for length of CD Array – Runs for every model tested, each with a different CD value

  - Defined the variable currentCD (currentCD = cdArray(g))

- Inner Loop

  - Set to run for length of ISP Array – Tests every thruster available

    - Define ISP to be tested (ISP = ispArray(t))

    - Define Efficiency of Thruster (etaT = efficiencyArray(t))

- Second Inner loop

  - Set to run for length of altitude array – tests every altitude

  - Calculates the Bus AreaRef for the given model (Using the marker from the outer loop to define the model)

  - Calculates density for that altitude – Using MSISE Model

  - Sets error, tolerance and initial Solar Array Area for final loop

  - Takes outputs from 3rd inner loop (Goes after the end of the 4th loop)

    - Stores Drag Force in Array

    - Calculates Propellant Mass

    - Calculates energy needed in eclipse

    - Calculates Required Battery Capacity

    - Calculates Battery Mass

    - Applies Mass penalty to undeployed solar array configurations

    - Calculates Solar Array Mass

    - Calculates Dry Mass

    - Calculates Wet Mass

    - Adds Mass and power data to arrays for data plotting

- Third Inner Loop – Solar Array Sizing

  - Takes inputs from Above loop

    - Initial Solar Array Area

    - Tolerance

    - Error (Difference between current and newly sized solar array)

  - Calculates the current Aref (Bus + Current Solar Array)

  - Calculates Drag force

  - Calculates Thrust Power

  - Calculates Total Power (Thrust + Bus Power)

  - Calculate Eclipse Ratio -> Sun ratio (1 – Eclipse Ratio)

  - Calculate Power Generation Required

  - Size New Solar Array based on Power Generation Required

  - Find Error between solar arrays

    - If error > tolerance restart loop

    - Eventually Converges at a solar array

Phase 3

Step 1: Power System Verification

- SIMULINK Model to prove sized power system is functional

- Basic System:

  - Draws constant Total Power

  - Verify that inputs maintain a DoD of 30% (Doesn’t drop below 70% Charge):

    - SA Area

    - Eta

    - Battery Capacity

- Dynamic Power:

  - Draws Max power during day

  - Draws Minimum Power during Night

  - (Use Switch Block)

- Advanced Active Power – Power is drawn dependant on thrust needed:

  - Takes Density from MSISE and applies it directly to calculate thruster force (Use Gain Block)

  - Use Add Blocks for Day and night

    - Day = Total Power + Gain

    - Night = Minimum Power + Gain

    - Attach these both to the Switch Block

- Add Magnetic Storm and Flux Variation in MSISE model

Step 2: Orbital Survival Verification

- Use DEMS to show that the satellite is able to survive in the atmosphere with sized thruster.

Sustainability

VLEO Orbit so investigate End Of Life disposal

Investigate how it complies with regulations.

Rubric

Use systems modelling methods to create an integrated systems model to critically evaluate design decisions.

- SIMULINK

- MATLAB

- Flowcharts

- Graphs

Make informed design trade off decisions by critically evaluating system performance.

- Alternate design configurations

  - Cuboid  x 2

  - Cylinder x 2

  - Hexagon

- Appropriate Rationale – Based on wet mass

Apply orbital dynamics and mission analysis methods in parallel with your system models to meet design requirements.

- Orbit Design Methods

  - SSO Repeating Ground Track

  - Orbit Mechanic evidence

  - SSO inclination graph

Synthesise information on industry trends into system design

- Air-breathing

- VLEO Operations – Flying closer to the Earth means significantly smaller, cheaper optical payloads, Flying as low as feasibly possible in our design

- Electric Propulsion – Ion Thrusters – High Specific Impulse with only a fraction of propellant mass. (Find the actual prop mass for the slides)

- Model-Based Systems Engineering (MBSE) – Move away from static spreadsheets and manual calculations to dynamic interconnected simulations

- Autonomous Safe Modes

Quantitative assessment of configuration alternatives feeding into trade off

Modelled evaluated and traded off within matlab.

Fidelity of system modelling and assumptions

MATLAB tradeoff shows this – Aref changing with design - Undeployed mass penalty.

Sources for the Atmosphere density variation

Dry mass margin:  +20% based on ESA Handbook

Power Margin: + 20% based on VLEO being a newly explored environment. Total ESA specifies a 20% power margin overall. Ensures 5 year requirement is met in harsh environment.

Propellant Margin: + 10% Margin – Allows unplanned manoeuvres, collision avoidance, and reclimb.

Clarity of Communication and Robustness of Conceptual Design

Make sure ALL requirements are met

Overall slides should be targeted to meet rubric

Speak clearly

Aim for 10 minutes at MOST