Primary Goal:

- Find Optimal Operational Altitude

  - Maximum of 600km

  - Minimise Wet Mass

- Imaging Data

  - Minimum – 5 years

- Only model payload operations phase

System Requirements:

- Resolution

  - 1m Ground Sample Distance (GSD)

  - Wavelength – 550 nm

  - Maximum off-nadir view angle – 45 degrees

- Revisit

  - Revisit time of less than 5 days

  - Target Latitudes between 40 and 80 degrees

- Orbit

  - Must be a repeating SSO

  - Local Time of Ascending Node of 10:30am

- Sensor

  - Signal-to-Noise Ratio greater than 100

- Duration

  - Minimum 5-year operational lifespan

Payload & Operation Assumptions

- Physical Size

  - Length is Twice optical aperture

- Focal Length

  - Within 3 to 15 times of OTA Length

- Mass Scaling

  - OTA Mass=146 D1.27

  - D : Aperture Diameter

- Operations:

  - Continuous Push-broom imaging

  - Imaging while in sunlight

Attitude

- Pointing:

  - Push Broom Mode

  - Achieved by rolling satellite

- Drag Profile

  - Assume constant orientation relative to velocity vector

- Atmosphere

  - MSISE Atmosphere Model

  - Assume conservative solar activity values

Power Subsystem Assumptions

- Base Power

  - Satellite Platform Requires constant 100W Power

  - Need extra for electric propulsion

- Generation

  - Triple Junction Solar Arrays

  - 28% Efficiency

  - 80% Packing density

  - Annual degradation rate of 2%

- Array Mass

  - 7.86 kg/m^2

- Storage

  - Li-Ion Batteries

    - Energy density – 150Wh/kg

- Voltages:

  - Main Bus – 5V

  - Solar Array – 120% of Bus (6V)

Mass, Volume & Structure Assumptions

- Structure

  - Assume structural mass accounts for 20% of dry mass

- Secondary Systems

  - Comms, ADCS, Thermal, OBDH

    - Accounts for 25% total dry mass

    - Accounts for 20% Satellite Volume

- Antenna

  - Assume Patch antenna (No moving parts)

    - Placed on Nadar face of platform